C and Embedded C

Interview Cheat Sheet

Read this the night before and again one hour before the interview.

How to answer any question (30 seconds) 1. Say the definition in one line. 2. Give a tiny example. 3. Say why it matters in embedded. Example: "volatile tells the compiler a value can change outside normal code flow, so it must read it from memory every time. I use it for hardware registers and flags changed in an ISR."

# 1. Top 30 one-line answers

| Question | Answer |
| --- | --- |
| Pointer? | A variable that stores a memory address. & = address of, * = value at. |
| Dangling pointer? | Points to memory that was freed or went out of scope. |
| Wild pointer? | Pointer never initialised, holds a random address. |
| NULL pointer? | Pointer holding 0, points to nothing. Always check before use. |
| void pointer? | Generic pointer without a type. Cast before dereferencing. |
| const vs #define? | const is a typed variable the compiler protects; #define is typeless text replacement. |
| static (local)? | Keeps its value between calls; lives for the whole program. |
| static (global/function)? | Visible only inside that .c file. |
| extern? | Declares a variable defined in another file. |
| volatile? | Value can change outside the code (hardware, ISR). Compiler must not cache it. |
| const volatile? | Read-only for code, changed by hardware (status register). |
| struct vs union? | Struct: members have separate memory. Union: all members share one memory. |
| Struct padding? | Gaps added for alignment. Order members large to small to save RAM. |
| Stack vs heap? | Stack: automatic, fast, for locals. Heap: manual (malloc/free), flexible, can leak. |
| Memory leak? | malloc without free; memory becomes unreachable. |
| malloc vs calloc? | calloc zeroes memory and takes (count, size). |
| .text .data .bss? | Code ¦ initialised globals ¦ zero-initialised globals. |
| Why globals are 0, locals garbage? | Startup code clears .bss; locals sit on the stack with old data. |
| What runs before main()? | Reset handler: set stack, copy .data to RAM, zero .bss, init clock, call main. |
| Endianness? | Byte order of multi-byte values. ESP32, AVR, most ARM are little-endian. |
| ISR rules? | Short, no delay, no printf, no malloc, shared data volatile, set flag and return. |
| Interrupt latency? | Time from the event to the first instruction of the ISR. |
| Polling vs interrupt? | Polling checks repeatedly (simple, wastes CPU); interrupt reacts to events (fast, saves CPU). |
| Race condition? | Result depends on timing between ISR and main using the same data. |
| Switch bounce? | Contacts make/break for 1 to 20 ms. Fix: RC filter or software timing. |
| Pull-up resistor? | Holds an input HIGH when nothing drives it; avoids a floating pin. |
| PWM? | Fast on/off switching; duty cycle sets average power. |
| Watchdog? | Hardware timer that resets the chip if software does not kick it in time. |
| UART / I2C / SPI? | UART: 2 devices, async. I2C: 2 shared wires, addresses. SPI: 4 wires, fast, chip select. |
| Why avoid malloc in firmware? | Fragmentation, unpredictable timing, tiny RAM. |

# 2. C language quick reference

## Type sizes (bytes) and ranges

| Type | PC | Uno (AVR) | ESP32 / STM32 | Range (signed) |
| --- | --- | --- | --- | --- |
| char | 1 | 1 | 1 | -128 to 127 |
| short | 2 | 2 | 2 | -32,768 to 32,767 |
| int | 4 | 2 | 4 | 16-bit: +/-32K 32-bit: +/-2.1 billion |
| long | 4 or 8 | 4 | 4 | +/-2.1 billion (4 bytes) |
| long long | 8 | 8 | 8 | +/-9.2 x 10^18 |
| float / double | 4 / 8 | 4 / 4 | 4 / 8 | 6-7 / 15-16 digits |
| pointer | 8 | 2 | 4 | holds an address |

Unsigned: 8 bit 0..255, 16 bit 0..65,535, 32 bit 0..4,294,967,295. Signed n-bit: -2^(n-1) to 2^(n-1) - 1.

Negative numbers: two's complement = invert bits, add 1.  -5 in 8 bit = 11111011.

## Format specifiers

| %d | int | %u | unsigned | %c | char |
| --- | --- | --- | --- | --- | --- |
| %ld | long | %lld | long long | %s | string |
| %f | float/double | %lf | double (scanf) | %p | pointer |
| %x | hex | %o | octal | %zu | size_t |

## Operator precedence (high to low)

Brackets  >  Unary (! ~ ++ -- * & sizeof)  >  * / %  >  + -  >  << >>  >  < <= > >=  >  == !=  >  &  >  ^  >  |  >  &&  >  ||  >  ?:  >  =

Remember: & is LOWER than ==, so write (x & 4) == 4. When in doubt, use brackets.

## Pointers

| Declaration | Meaning (read right to left) |
| --- | --- |
| int *p | p is a pointer to int |
| const int *p | pointer to constant int: cannot change *p |
| int *const p | constant pointer: cannot move p |
| int **pp | pointer to pointer to int |
| int *arr[5] | array of 5 pointers to int |
| int (*fp)(int, int) | pointer to function(int, int) returning int |
| arr[i] == *(arr + i) | indexing is pointer arithmetic; p + 1 moves by sizeof(type) |

Mnemonic: & = WHERE (address), * = WHAT (value there).

## Storage classes

| Class | Scope | Lifetime | Note |
| --- | --- | --- | --- |
| auto | block | block | default for locals, garbage value |
| register | block | block | hint only, modern compilers ignore |
| static | block or file | whole program | default value 0, private to file |
| extern | whole program | whole program | declared here, defined elsewhere |

## Strings and memory rules

- String = char array ending with '\0'. "hello" needs 6 bytes. strlen = 5, sizeof = 6.

- Compare strings with strcmp (0 means equal), never ==.

- char s[] = "hi" is modifiable. char *s = "hi" points to read-only text.

- malloc: check for NULL, use n * sizeof *p, free once, set pointer to NULL after free.

- realloc: assign to a temporary pointer first, otherwise a failure leaks the old block.

- Macros: put every parameter and the whole expression in brackets. Header files: add include guards.

- Call by value: functions get copies. Pass a pointer to change the caller's variable.

# 3. Embedded C quick reference

## Bit manipulation

| Goal | Code | Mnemonic |
| --- | --- | --- |
| Set bit n | x ¦= (1U << n); | SET = OR |
| Clear bit n | x &= ~(1U << n); | CLEAR = AND-NOT |
| Toggle bit n | x ^= (1U << n); | TOGGLE = XOR |
| Check bit n | (x >> n) & 1 | CHECK = AND |
| Read field | (x >> pos) & mask | shift down, then mask |
| Write field | x = (x & ~(mask << pos)) ¦ (val << pos); | clear it, then set it |

Shift left n = multiply by 2^n.   Shift right n = divide by 2^n (unsigned).   Use unsigned types and 1U for bit work.

12 & 10 = 8     12 | 10 = 14     12 ^ 10 = 6     ~12 = -13     12 << 1 = 24     12 >> 1 = 6

## volatile: needed for 3 things

Mnemonic RIS:  Register (hardware)   |   Interrupt-changed flag   |   Shared between tasks.

volatile does NOT make access atomic. For multi-byte shared data, disable interrupts briefly.

## Memory

| Section | Holds | Where |
| --- | --- | --- |
| .text | code | Flash |
| .rodata | const data, string literals | Flash |
| .data | initialised globals / statics | Flash copy -> RAM |
| .bss | zero or uninitialised globals | RAM (zeroed at startup) |
| Heap | malloc, grows up | RAM |
| Stack | locals, return addresses, grows down | RAM |

Startup order: vector table -> Reset handler -> set stack -> copy .data -> zero .bss -> init clocks -> main().

Little-endian: 0x12345678 stored as 78 56 34 12.   Padding: struct {char; int; char} = 12 bytes, reorder to {int; char; char} = 8 bytes.

Stack overflow causes: deep recursion, large local arrays. Fix: make buffers static, avoid recursion, increase task stack.

## Interrupts

Flow: event -> flag -> finish instruction -> save context -> vector table -> ISR -> restore -> resume.

ISR mnemonic: SHORT, SIMPLE, no SLEEP (delay), no PRINT. Set a flag, return, do the work in main.

- Shared variables: volatile. Multi-byte reads on 8-bit chips: noInterrupts() ... interrupts().

- ESP32 ISRs need IRAM_ATTR. Uno external interrupts only on pins 2 and 3.

- Clear the interrupt flag if the hardware requires it, or the ISR fires again forever.

- Debounce button interrupts (ignore triggers within about 50 ms).

## GPIO

- Active-low button: INPUT_PULLUP, pressed = LOW. Input floating = random values.

- Push-pull drives high and low. Open-drain pulls low only, needs pull-up (I2C).

- LED resistor: R = (Vsupply - Vled) / I.  Example: (3.3 - 2.0) / 0.01 = 130 ohm, use 220 ohm.

- Motors, relays, solenoids: use a transistor or driver with flyback diode, never a bare GPIO.

- ESP32 GPIO 34 to 39: input only, no internal pull-ups. ESP32 / ESP8266 pins are 3.3 V only.

# 4. Formulas to remember

| What | Formula and example |
| --- | --- |
| Timer tick rate | f_timer = f_clock / prescaler. 16 MHz / 64 = 250 kHz (4 us per tick) |
| Timer compare value | ticks = time x f_timer, compare = ticks - 1. 1 ms: 250 ticks, OCR = 249 |
| PWM duty cycle | duty % = ON time / period x 100. Average voltage = duty x Vcc |
| PWM 8-bit | 256 levels (0 to 255). value 128 is about 50% |
| ADC voltage | V = count x Vref / maxCount. Uno: 512 x 5 / 1023 = 2.5 V |
| ADC step size | Vref / 2^N. 10-bit at 5 V = 4.9 mV; 12-bit at 3.3 V = 0.8 mV |
| ADC counts | Uno / ESP8266: 0 to 1023 (10-bit). ESP32: 0 to 4095 (12-bit) |
| UART byte time | 10 bits / baud. 9600 baud = 1.04 ms per byte; 115200 = 87 us |
| LM35 temperature | degC = volts x 100 (10 mV per degree) |
| Average current | (I_active x t_active + I_sleep x t_sleep) / t_total |
| millis() safe compare | now - previous >= interval (works across wrap-around, unsigned) |
| Servo | 50 Hz (20 ms), pulse 1 ms to 2 ms for full travel |

# 5. Protocols side by side

|  | UART | I2C | SPI |
| --- | --- | --- | --- |
| Wires | TX, RX, GND | SDA, SCL (+ pull-ups) | SCK, MOSI, MISO, CS |
| Devices | 2 only | many (7-bit address) | many (one CS each) |
| Clock | none, same baud | master clock | master clock |
| Speed | 9600 to 1M+ baud | 100 / 400 kHz | 1 to 50+ MHz |
| Duplex | full | half | full |
| Used for | PC, GPS, Bluetooth | sensors, OLED, RTC | SD card, TFT, flash |
| Gotcha | cross TX/RX, same baud, common GND | pull-ups 4.7k, address clash | mode (CPOL/CPHA), CS |

UART frame 8N1: start bit, 8 data bits (LSB first), no parity, stop bit.   I2C default pins: Uno A4/A5, ESP32 GPIO 21/22.

Choose: PC or module with serial port = UART.  Many slow sensors on few pins = I2C.  High speed data = SPI.

# 6. Design patterns in one line each

| Pattern | Remember |
| --- | --- |
| Ring buffer | head writes, tail reads, wrap with (i + 1) & (SIZE - 1); empty: head == tail; full: next(head) == tail. |
| State machine | enum for states + switch; each state checks an event or millis() and sets the next state; never blocks. |
| Watchdog | Kick from main loop only when healthy, never from an ISR. |
| Deep sleep | Wake, measure, send, sleep. Restarts at setup(); keep data in RTC memory. |
| RTOS task | Own stack and priority; use vTaskDelay, queues and mutexes for sharing. |
| Callback | Driver stores a function pointer; calls it when the event happens. |
| Super-loop | while(1) { read inputs; update state; write outputs; } |

# 7. Code you must be able to write from memory

## Bit macros

#define SET_BIT(x, n)     ((x) |=  (1U << (n)))

#define CLEAR_BIT(x, n)   ((x) &= ~(1U << (n)))

#define TOGGLE_BIT(x, n)  ((x) ^=  (1U << (n)))

#define CHECK_BIT(x, n)   (((x) >> (n)) & 1U)

## Power of 2, count set bits, swap

int is_pow2(unsigned n) { return n && !(n & (n - 1)); }

int count_bits(unsigned n) {

int c = 0;

while (n) { n &= n - 1; c++; }      // removes lowest 1 each loop

return c;

}

void swap(int *a, int *b) { int t = *a; *a = *b; *b = t; }

// no temp: a ^= b; b ^= a; a ^= b;

## Strings

int my_strlen(const char *s) {

int n = 0;

while (*s++) n++;

return n;

}

void reverse(char *s) {

int i = 0, j = my_strlen(s) - 1;

while (i < j) { char t = s[i]; s[i++] = s[j]; s[j--] = t; }

}

## Endianness check

uint32_t x = 1;

if (*(uint8_t *)&x == 1) puts("little-endian");

else                     puts("big-endian");

## Register access and function pointer

#define REG  (*(volatile uint32_t *)0x40020014)   // example address

REG |= (1U << 5);

typedef void (*Callback)(void);

Callback cb = my_handler;

cb();

## Ring buffer (put and get)

bool rb_put(RB *r, uint8_t b) {

uint8_t next = (r->head + 1) & (SIZE - 1);

if (next == r->tail) return false;      // full

r->buf[r->head] = b;  r->head = next;  return true;

}

bool rb_get(RB *r, uint8_t *b) {

if (r->head == r->tail) return false;   // empty

*b = r->buf[r->tail];  r->tail = (r->tail + 1) & (SIZE - 1);  return true;

}

## Non-blocking blink and debounce skeleton

if (millis() - previous >= 1000) { previous = millis(); toggle(); }

if (reading != lastReading) lastChange = millis();

lastReading = reading;

if (millis() - lastChange > 30 && reading != stable) {

stable = reading;                       // accepted: real press or release

}

# 8. Tricky output questions

| Code | Result | Why |
| --- | --- | --- |
| 7 / 2 | 3 | integer division |
| -7 / 2 and -7 % 2 | -3 and -1 | truncates toward zero; remainder takes the sign of the left side |
| sizeof("hello") strlen("hello") | 6 5 | sizeof counts \0 |
| sizeof('a') in C | 4 (int size) | a character constant is an int in C |
| unsigned char c = 255; c++; | 0 | unsigned wraps around |
| if (-1 < 1u) | false | -1 converts to a huge unsigned value |
| 0.1 + 0.2 == 0.3 | false | binary floating point is approximate |
| int x = 012; | x = 10 | leading 0 means octal |
| 5 & 3 5 ¦ 3 5 ^ 3 ~5 | 1 7 6 -6 | bit by bit |
| 5 && 3 !5 !0 | 1 0 1 | logical operators give 0 or 1 |
| int a = 5; b = a++; c = ++a; | b = 5, c = 7 | post uses then adds; pre adds then uses |
| printf("%d", printf("hi")); | hi2 | printf returns the number of characters printed |
| sizeof(arr) inside a function with int arr[] | pointer size | array parameter decays to a pointer |
| char *p = "hi"; p[0] = 'H'; | undefined behaviour | string literal is read-only |
| int arr[5]; arr[5] = 1; | undefined behaviour | out of bounds, C does not check |
| 1 << 31 on a 32-bit signed int | undefined behaviour | shifts into the sign bit; use 1U << 31 |
| i = i++ + ++i; | undefined behaviour | modified twice without a sequence point |
| #define SQ(x) x*x SQ(2+3) | 11 | 2+3*2+3; brackets missing |

# 9. Mnemonics

| Topic | Memory trick |
| --- | --- |
| Bit operations | Set = OR, Clear = AND NOT, Toggle = XOR, Check = AND. |
| Pointers | & = where, * = what. Read declarations right to left. |
| volatile (RIS) | Register, Interrupt flag, Shared data. |
| ISR | Short, Simple, no Sleep, no Print. |
| Memory sections | Text = code (flash), Data = given values, BSS = Zeros at start. |
| Heap vs stack | Stack = automatic hotel room; heap = rented hall you must return. |
| SPI wires | SCK clock, MOSI master out, MISO master in, CS choose. |
| I2C | Two wires, many names (addresses), pull-ups required. |
| Debounce | Wait until the signal stays steady for 20 to 50 ms. |
| Endianness | Little-endian: little end (lowest byte) first. |

# 10. Interview day checklist

- Know your projects cold: purpose, hardware, sensors, protocols, the hardest bug, what you would improve. Practise a 2-minute explanation for each.

- Say "I don't know, but I would find out by checking the datasheet or testing it" instead of guessing. Honesty plus method impresses.

- Think aloud when solving a coding question: state the approach, write the code, test with an edge case (0, 1, maximum, empty input).

- Draw diagrams when asked about memory, interrupts or protocols. Diagrams show understanding.

- Link answers to your own work: debouncing in your door lock, ADC in your soil sensor, I2C in your OLED projects, interrupts in your traffic signal.

- Ask one good question at the end: "What does a typical day or first project look like for this role?"

- Sleep well, carry a pen and paper, and be 15 minutes early.

Last-minute rule If you remember only 5 things: pointers, bit manipulation, volatile, memory sections and ISR rules. These come up in almost every embedded interview.